SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE:  CSA0713                    COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I PERARASU.S(192524024) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

nnexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

5. A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

ANALYTICAL PROBLEM SOLVING
Detailed Step-by-Step Solutions

1. CRC (Cyclic Redundancy Check)

Given Message = 11100011
Generator Polynomial = x^3 + 1
Generator Binary = 1001
Degree = 3, therefore append three zeros.

Step 1: Append zeros:
11100011 → 11100011000

Step 2: Perform modulo-2 (XOR) long division using divisor 1001. At each step, whenever the leftmost bit is 1, XOR with 1001; otherwise XOR with 0000 and bring down the next bit. After completing the division, the final remainder is 100.

Step 3: Append the remainder to the original message.
Transmitted codeword = 11100011 + 100 = 11100011100.

Error case: If the leftmost transmitted bit is inverted, the receiver divides the received codeword by 1001. The remainder is non-zero (not 000), so the CRC check fails and the receiver detects an error.

2(a). Throughput

Bandwidth = 10 Mbps
Frames/minute = 12,000
Frame size = 10,000 bits

Bits transmitted per minute = 12,000 × 10,000 = 120,000,000 bits.

Throughput = 120,000,000 / 60 = 2,000,000 bps = 2 Mbps.

Efficiency = (2/10) × 100 = 20%.

2(b). Shannon Channel Capacity

Formula: C = B log2(1 + S/N)

B = 3000 Hz
S/N = 3162

C = 3000 × log2(3163)
≈ 3000 × 11.63
≈ 34,890 bps ≈ 34.9 kbps.

3. Hamming (12,8) Error Correcting Code

Hamming (12,8) uses 8 data bits and 4 parity bits (positions 1,2,4,8).

Steps:
1. Place data bits in remaining positions.
2. Calculate parity bits.
3. Transmit 12-bit codeword.
4. At the receiver, recompute parity.
5. The syndrome indicates the incorrect bit position.
6. Flip that bit to recover the original data.

Advantages: Corrects one-bit error, detects single-bit corruption, avoids retransmission, improves reliability in wireless medical monitoring.

4(a). IPv4 Subnetting

Given Network = 172.16.10.0/24
Subnet Mask = /27

Borrowed bits = 27 - 24 = 3
Subnets = 2^3 = 8

Host bits = 5
Addresses per subnet = 2^5 = 32
Usable hosts = 30

Subnet ranges:
0-31, 32-63, 64-95, 96-127, 128-159, 160-191, 192-223, 224-255

172.16.10.78 lies in subnet 172.16.10.64/27.
Network Address = 172.16.10.64
Broadcast Address = 172.16.10.95
Usable Hosts = 172.16.10.65 to 172.16.10.94.

172.16.10.95 is the broadcast address, so it belongs to the same subnet but cannot be assigned to a host.

4(b). IPv6

Address: 2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed form: 2001:db8::ff00:42:8329

Expanded form: 2001:0db8:0000:0000:0000:ff00:0042:8329

Using /64 prefix, network prefix = 2001:0db8:0000:0000::/64

Host bits = 64
Possible host addresses = 2^64 = 18,446,744,073,709,551,616

5. Flow Control Analysis

Problem identified: Flow Control.

The server sends data at 10 Gbps while many clients receive at only 5–100 Mbps. Because receivers process data more slowly, their buffers fill up and overflow. Packets are dropped, acknowledgements are delayed, TCP retransmits lost packets, network congestion increases, and students experience slow or incomplete downloads.

Solution: TCP flow control using the sliding window mechanism adjusts the sender's transmission rate according to the receiver's advertised window, preventing buffer overflow and improving reliability.

Additional Notes 1

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.

Additional Notes 2

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.

Additional Notes 3

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.

Additional Notes 4

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.

Additional Notes 5

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.

Additional Notes 6

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.

Additional Notes 7

This page can be used for handwritten CRC long division, subnet calculations, or examination notes. Key formulas:
- Throughput = Data transferred / Time
- Shannon Capacity = B log2(1+S/N)
- Subnets = 2^(borrowed bits)
- Hosts = 2^(host bits)-2
- IPv6 /64 leaves 64 host bits.
- Hamming code corrects one-bit errors.
- CRC detects transmission errors using polynomial division.